U. S. DEPARTMENT OF THE INTERIOR

U. S. GEOLOGICAL SURVEY

Preliminary Geologic Map of the New Castle

Quadrangle, Garfield County, Colorado(1)

by

MORRIS W. GREEN, GEORGE M. FAIRER, AND RALPH R. SHROBA(2)

Open-File Report 93-310

This report is preliminary and has not been reviewed for

conformity with U.S. Geological Survey editorial standards

or with the North American Stratigraphic Code.  Any use of

trade, product, or firm names is for descriptive purposes

only and does not imply endorsement by the U.S.  Government.

Although these data have been used by the U.S. Geological

Survey and have been successfully imported into data base

programs, no warranty, expressed or implied, is made by the

USGS as to how successfully or accurately the data can be

imported into any specific application software running on

any specific hardware platform.  The fact of distribution

shall not constitute any such warranty, and no

responsibility is assumed by the USGS in connection

therewith.

1993

(1)  This database of the New Castle 7.5 minute quadrangle,

originally released as U.S Geological Survey Open-File

Report 93-310 has been updated to include a digitized

database.  The digitized version is also identifed as OFR

93-310.  The Digital database is in ARC/INFO Format.

Although this database has been subjected to rigorous review

and is substantially complete, the USGS reserves the right

to revise the data pursuant to further analysis and review.

Furthermore, it is released on condition that neither the

USGS nor the United States Government may be held liable for

any damages resulting from its authorized or unauthorized

use.

This digital geologic map does not include a digital

(DRG) topographic data file as it is currently unavailable.

When the DRG database becomes available it can be combined

with the geologic database.

The geologic database can be downloaded via 'anonymous

ftp' from a USGS system named greenwood.cr.usgs.gov

(136.177.48.5).  The files are located in a directory named

/pub/open-file-reports/ofr-93-310.

The database manager is: Ralph R. Shroba

(303) 236-1293

U.S. Geological Survey

Box 25046, Mail Stop 913

Denver, CO 80225

This geologic map was prepared as part of a study of

digital methods and techniques as applied to complex

geologic maps.  The geologic map was scanned from the

original greenline sheets used to prepare the published

Open-File map of the New Castle quadrangle (Green and

others, 1993).  Consequently, the digital version is at

1:24,000 scale using the Polyconic map projection parameters

of the topographic base map.  Stable base contact prints of

the greenline sheets were scanned on an Ana Tech Eagle 408ET

scanner to produce a .tif file.  These files were then

transferred to a Sun SPARCstation 20 computer where they

were converted into ARC/INFO format.  Each vector and

polygon was given attributes derived from the original 1993

geologic map using ALACARTE (Wentworth and Fitzgibbon, 1991)

software.

This database was developed on a SPARCstation 20

computer system using ARC/INFO 7.0.2 software.  The lineset

and shadeset files are coded for a HP DesignJet 650C plotter

using HPGL2 software.

List of files:

alcgeol.e00(a)  symbol set

alcwrg.e00(a)   line set

ctr269.e00(b)   shade set

fnt003.e00(c)   font set

loadsk.aml      load program

loess.e00       shade set

mapbar.aml(b)   mapbar program

nc.box          index box program

nc.text         index text

nc8.e00         line and polygon data set

ncex.aml        map assembly program

ncl.e00         loess polygon data set

nclb.box        loess index box program

ncstr.e00       structure data set

wyo.e00(c)      text set

nc.doc          text (this file)

README.SK       readme text

seals.e00(c)    logo polygon set

(a) T.T. Fitzgibbon

(b) W.R. Beaman

(c) G.R. Green

The authors acknowledge the assistance of G.N. Green in

preparation of the digitized version of this geologic map.

Line Types and Attributes

Line Types             ALCWRG

Contact, certain         25

Contact, concealed       29

Normal fault, certain    1

Normal fault, inferred   3

Normal fault, concealed  5

Anticline                31

Syncline                 31

Monocline                31

Water boundary           25

Polygon Types and Attributes

P-types                ALCWRG

Qfp                      8

Qfy                      1

Qty                      38

Qtm                      10

Qto                      34

Qtt                      40

Qfo                      217

Qp                       218

Qbb                      53

QTba                     67

Qac                      3

Qc                       2

Qsw                      5

Qls                      4

Qta                      36

Qlo                      37

Tw                       62

Kmv                      78

Km                       80

Kd                       82

Jm                       223

Je                       224

Trc                      235

TrIPsb                   236

PIPm                     228

(2) Denver, Colorado

Preliminary Geologic Map of the New Castle Quadrangle,

Garfield County, Colorado

By Morris W. Green, George M. Fairer, and Ralph R. Shroba

CORRELATION OF MAP UNITS

Refer to non-digitized version of this open-file report for

a diagram showing correlation of map units

DESCRIPTION OF MAP UNITS

[Surficial deposits shown on the map are estimated to be at

least 3 ft thick.  Fractional map symbols (for example,

Qlo/Qto) are used where loess mantles older surficial

deposits and the underlying deposits have been identified.

Artificial fills and thin, discontinuous colluvial deposits

and residual material on bedrock were not mapped.

Artificial fills consist mainly of compacted fill material

beneath segments of U.S. Highway 6, Interstate 70, and the

nearby tracks of the Denver and Rio Grande Western Railroad,

and uncompacted fill in several small mine dumps.  Also not

mapped are several elongate areas above mined coal beds on

the north side of the Grand Hogback, near Burning Mountain

and Coal Ridge, where the ground surface is highly oxidized,

fractured, and prone to subsidence (Stover and Soule, 1985).

Divisions of Pleistocene time correspond to those of

Richmond and Fullerton (1986).  Age assignments for

surficial deposits are based chiefly on the degree of

modification of original surface morphology, height above

stream level, and degree of soil development.  Age

assignments for units Qtt, Qbb, and QTba are based chiefly

on a regional rate of stream incision of about 45 ft/100 ka

and on a regional rate of tectonic uplift of about

600 ft/Ma.  The incision rate is based on an average of

three values for stream incision, since the deposition of

the 620-ka Lava Creek B volcanic ash, of about 300 ft along

the Colorado River near the east end of Glenwood Canyon

(Izett and Wilcox, 1982), about 290 ft along the Roaring

Fork River near Carbondale, Colorado (Piety, 1981), and

about 260-280 ft along the White River near Meeker, Colorado

(J.W. Whitney, oral commun., 1992; Whitney and others,

1983).  The rate of tectonic uplift of about 6000 ft/10 Ma

was determined for the Derby Peak fauna, in the Flat Tops

area (Colman, 1985), which is about 30 mi northeast of the

quadrangle.  Soil-horizon designations are those of the Soil

Survey Staff (1975) and Guthrie and Whitty (1982).  Most of

the surficial deposits are calcareous and contain different

amounts of primary and secondary calcium carbonate; stages

of secondary calcium carbonate morphology are those of Gile

and others (1966).  Grain sizes given for surficial deposits

are based on visual estimates and follow the modified

Wentworth grade scale (American Geological Institute, 1982).

In descriptions of surficial map units, the term clasts

refers to the fraction greater than 0.08 in. (2 mm) in

diameter, whereas the term matrix refers to the finer

material.  Dry matrix colors of the surficial deposits were

determined by comparison with Munsell Soil Color Charts

(Munsell Color, 1973).  The colors of the surficial deposits

correspond to those of the sediments and(or) bedrock from

which they were derived.  Surficial deposits derived from

non-red sediments and bedrock are commonly light brownish

gray (2.5Y 6/2), pale yellow (2.5Y 7/4), light gray (10YR

7/2), very pale brown (10YR 7/3, 8/3, 7/4, and 8/4), pale

brown (10YR 6/3), light yellowish brown (10YR 6/4), light

brown (7.5YR 6/4), and pink (7.5YR 7/4).  Those derived from

red sediments and bedrock are commonly light reddish brown

(5YR 6/4 and 2.5YR 6/4), reddish brown (5YR 5/4 and 2.5YR

5/4), reddish yellow (5YR 6/6), light red (2.5YR 6/6), and

red (2.5YR 5/6).]

Alluvial deposits-Silt, sand, and gravel in flood plains,

stream channels, and terraces along the Colorado River and

its major tributaries, in alluvial fans on flood plains and

terraces, and in pediment deposits on gently sloping

surfaces cut on bedrock

Qfp  Flood plain and stream channel deposits (Holocene and

late Pleistocene)-

Chiefly clast-supported, slightly bouldery, pebble- and

cobble-gravel in a sand matrix that is locally overlain by

gravelly sand to sandy silt.  Along the Colorado River, the

upper 3-4 ft of the flood plain deposits is commonly clean

or humic and slightly silty, very fine to medium sand that

locally contains a minor amount of pebbles and cobbles.

Poorly to moderately well sorted and poorly to well bedded.

Clasts are commonly subangular to rounded; their lithologies

reflect those of the bedrock in the upstream areas.  The

unit occurs along the Colorado River and its major

tributaries.  Low-lying areas are prone to periodic

flooding.  Deposits along tributary streams may contain more

sand and silt than those along the Colorado River.  The unit

locally includes small alluvial-fan deposits (Qfy), low

terrace deposits that are commonly less than 15 ft above

modern stream level, and sheetwash deposits (Qsw), and

locally may include some organic-rich deposits.  A thick

(>45 ft) sequence of flood plain deposits, which is exposed

in gullies along West Elk Creek (NW sec. 16, T. 5 S.,

R. 91 W.), consists of interbedded sand and silt.  This

thick sequence of interbedded sand and silt may have been

produced in part by damming of West Elk Creek by one or more

former alluvial fans from intermittent tributary drainages

downstream of the sequence.  The upper part of the unit may

be a complex of cut-and-fill deposits of Holocene and late

Pleistocene(?) age.  The lower part of the unit is probably

equivalent, at least in part, to  the younger terrace

alluvium (Qty).  The unit is tentatively correlated with

deposits in terrace T8 of Piety (1981) along the Roaring

Fork River between Glenwood Springs and Carbondale, Colo.

Thickness along the Colorado River 15 ft and more near the

western quadrangle boundary and greater than 45 ft near the

eastern quadrangle boundary (Colorado Highway Department,

unpublished data); maximum exposed thickness 45 ft along

West Elk Creek; maximum thickness along the Colorado River

may be about 60 ft

Qfy  Younger fan alluvium (Holocene and latest Pleistocene)-

Mostly poorly sorted, clast- and matrix-supported, slightly

bouldery pebble- and cobble-gravel in a silty sand matrix,

and locally pebbly and cobbly silty sand that contains thin

(4-16 in.) lenses of sand, pebble gravel, and cobbly pebble

gravel.  At the mouth of Alkali Creek, the unit is mostly

very thin-bedded (0.2-0.8 in.) sandy silt.  The unit locally

contains boulders as long as 12 ft.  Some of the larger

boulders were probably deposited by debris flows.  Nonbedded

to poorly bedded; beds are commonly less than 3 ft thick.

Clasts are commonly angular to subangular sandstone north of

the Colorado River and are angular to subangular sandstone

and angular to subrounded basalt south of the Colorado

River.  The unit is undissected and was deposited chiefly by

small intermittent streams graded to the flood plains of

modern streams (Qfp) and locally to the tops of terraces

that are underlain by younger terrace alluvium (Qty).

Locally includes valley-fill deposits of intermittent

streams, debris-flow deposits, sheetwash deposits (Qsw), and

colluvium (Qc).  Exposed thickness 10-55 ft; maximum

thickness probably about 80 ft

Qty  Younger terrace alluvium (late Pleistocene)-Stream

alluvium that underlies terrace remnants that are about

40 ft above the Colorado River, Divide Creek, and Elk Creek.

Along the Colorado River, the lower part of the unit

consists mostly of a poorly sorted, clast-supported, pebble-

and cobble-gravel in a sand matrix.  Where deposited by

minor tributary streams, the upper part of the unit commonly

consists of 2-23 ft of silty sand that contains lenses and

thin (as much as 20 in.) beds of cobbly pebble gravel and

pebbly sand.  Clasts in the lower part of the unit are

commonly subrounded to rounded and are derived from a

variety of sedimentary, igneous, and metamorphic rocks in

the upstream areas.  Where deposited by minor tributary

streams, the clasts in the upper part of the unit are basalt

and sandstone on the south side of the Colorado River and

are almost entirely sandstone on the north side of the

river.  Along Divide Creek, the unit is mostly a poorly

sorted, clast-supported, bouldery, cobble and pebble gravel

in a sand matrix.  Near the mouth of Divide Creek, the unit

is a gravelly silty sand that contains lenses of cobbly

pebble gravel.  Clasts are chiefly subrounded to rounded

basalt.  Along Elk Creek, the unit is poorly exposed, but it

appears to consist mostly of poorly sorted, clast-supported,

cobble and pebble gravel in a sand matrix.  The unit along

the Colorado and its major tributaries is overlain by about

6-10 ft of loess (Qlo) and by younger fan alluvium (Qfy).

Unit Qty is probably equivalent in part to outwash of the

Pinedale glaciation, which is about 12-35 ka (Richmond,

1986, chart 1A).  Unit Qty is tentatively correlated with

deposits in terraces T7 and T6 of Piety (1981) along the

Roaring Fork River between Glenwood Springs and Carbondale,

Colo., and with deposits in terraces A and B of Bryant

(1979) farther upstream between Woody Creek and Aspen, Colo.

Exposed thickness 50 ft along the Colorado River and 15 ft

along

Divide Creek; maximum thickness along the Colorado River

possibly about 100 ft

Qtm   Intermediate terrace alluvium (late Pleistocene)--

Alluvium that underlies small terrace remnants about

70-80 ft above West Elk and Elk Creeks.  The unit is poorly

exposed, but it appears to consist of poorly sorted, clast-

supported, slightly bouldery, pebble- and cobble-gravel in a

sand matrix.  Clasts are mostly subangular to rounded

sandstone, quartzite, granite (or gneiss), limestone,

conglomerate, and siltstone.  The unit is locally weakly

cemented by fine-grained calcium carbonate and is locally

overlain by loess (Qlo) and by thin, unmapped deposits of

slopewash (Qsw) or colluvium (Qc).  The unit is inferred to

be intermediate in age between the younger and older terrace

alluviums (Qty and Qto) and may be of middle or early

Wisconsin age (about 35-65 or 65-80 ka; Richmond and

Fullerton, 1986, fig. 2).  Thickness probably about 15-30 ft

Qto   Older terrace alluvium (middle Pleistocene)--Stream

alluvium that underlies terrace remnants about 130-150 ft

above the Colorado River, about 130 ft above Divide,

Garfield, and Elk Creeks, and about 150 ft above West Elk

Creek.  Along the Colorado River, the unit mostly is a

poorly to moderately well sorted, clast-supported, bouldery

pebble- and cobble-gravel in a sand matrix that (1) is

overlain by about 23-50 ft of thin-bedded (0.5-6 in.),

slightly silty sand and thin (2-10 in.) lenses of pebbly

sand and sandy pebble gravel, or (2) grades upward into

slightly cobbly pebble gravel and pebbly sand in the upper

6-10 ft.  Locally the unit is a pebble gravel that is

overlain by about 10 ft of thin bedded, sandy silt that

contains thin (2-14 in.) lenses of sand and sandy pebble

gravel.  Locally the lower 3 ft of the unit is cemented by

fine-grained calcium carbonate.  Clasts are chiefly

subrounded to rounded sandstone, gneiss, quartzite, basalt,

granodiorite(?), limestone, and dolomite deposited by the

Colorado River.  Locally, the clasts in the upper part of

the unit were deposited by streams tributary to the

Colorado.  They are basalt and a minor amount of sandstone

near the mouths of Divide Creek and Garfield Creek and are

sandstone near the mouths of intermittent tributary streams

that head on the south flank of the Grand Hogback.  Along

Garfield Creek, the unit is mostly a poorly sorted, clast-

supported, bouldery, pebble- and cobble-gravel in a sand

matrix.  Clasts are chiefly subrounded to rounded basalt.

Some of the clasts are as long as 3.5 ft.  Along Divide

Creek the unit is mostly a poorly sorted, clast-supported,

bouldery, pebble- and cobble-gravel in a sand matrix.  The

unit locally contains a few thin (10-14 in.) lenses of sand

and sandy pebble gravel.  Clasts are chiefly subrounded to

rounded basalt and a minor amount of sandstone.  Locally the

clasts in the upper part of the unit are chiefly subangular

to subrounded sandstone and a minor amount of basalt.  Some

of the basalt clasts in the lower part of the unit are as

long as 5 ft.  Along Elk and West Elk Creeks, the unit is

mostly a poorly to moderately well sorted, clast-supported,

cobbly pebble gravel in a sand matrix.  The unit locally

contains lenses and beds of pebbly sand and sandy silt that

are about 2-40 in. thick and rare sandstone boulders, some

as long as 5.5 ft.  Locally some of the beds are weakly

cemented by fine-grained calcium carbonate and form small

ledges that are as much as 3 ft thick.  Clasts are chiefly

subrounded to rounded sandstone, quartzite, granite (or

gneiss),

limestone, conglomerate, and siltstone.  The unit along the

Colorado and its major tributaries is locally mantled by

8-12 ft of loess (Qlo) and by older fan alluvium (Qfo) and

slopewash deposits (Qsw).  Locally there is a buried soil

that is formed in the upper part of the unit.  The soil has

an argillic B horizon, 24 in. thick, that is formed in silty

alluvium or in the overlying loess.  It overlies a stage III

K horizon, 48 in. thick, that is formed in the upper part of

the underlying gravel.  The morphologic development of the

soil suggests that the unit is of Bull Lake age (Shroba,

1989) and may be about 140-150 ka (Pierce and others, 1976;

Pierce, 1979) or about 130-300 ka (late middle Pleistocene;

Richmond, 1986, chart 1A).  Locally the buried soil is

formed in the lower of two loess sheets that locally mantle

the unit.  The unit is tentatively correlated with deposits

in terraces T5 and T4 of Piety (1981) along the Roaring Fork

River between Glenwood Springs and Carbondale, and with

deposits in terrace C of Bryant (1979) farther upstream

between Woody Creek and Aspen.  Exposed thickness along the

Colorado River 35-120 ft, maximum thickness probably about

130 ft.  Exposed thickness 10 ft along Garfield Creek, 60 ft

along Divide Creek, 12-20 ft along Elk Creek, and 50 ft

along West Elk Creek.  Maximum thickness along these creeks

possibly about 100 ft

Qtt   Oldest terrace alluvium (middle Pleistocene)--Stream

alluvium that underlies small terrace remnants that are

about 220, 250, and 320 ft above the Colorado River, about

320 ft above Elk Creek near its confluence with East Elk

Creek, and about 320 ft above Garfield Creek near its

confluence with the Colorado River.  Along the Colorado

River and Elk Creek, the unit is mostly a poorly sorted,

clast-supported, slightly bouldery, pebble- and cobble-

gravel in a sand matrix.  It locally consists of thin

(0.2-14 in.) lenses and beds of silt, sandy silt, silty

sand, and sandy pebble gravel.  The unit locally grades

upward into about 12-20 in. of moderately well sorted,

clast-supported, pebble gravel in a sand matrix that is

overlain by about 20-30 in. of non-pebbly to slightly

pebbly, slightly silty sand.  Clasts are mostly subrounded

to rounded and are derived from a variety of sedimentary,

igneous, and metamorphic rocks in the upstream areas.

Locally along the Colorado River, the upper part of the unit

was deposited by tributary streams and is composed of

angular to subrounded sandstone or basalt clasts in a sand

matrix.  A stage III K soil horizon, 24-36 in. thick, is

locally present in the top of the unit and the lower 3-6 ft

of the unit is locally cemented by fine-grained calcium

carbonate.  Along Garfield Creek, the unit is mostly a

poorly sorted, clast-supported, bouldery, basalt-rich gravel

that contains a minor amount of sandstone and quartzite

clasts.  The basalt clasts are subrounded to rounded and are

as long as 6 ft.  The unit along the Colorado River and its

major tributaries is locally mantled by 6-13 ft of loess

(Qlo) and locally by about 15-30 ft of sandstone-rich older

fan alluvium (Qfo), which is overlain by loess.  The loess

mantle locally consists of two or more sheets.  The unit may

be correlative in part with other terrace deposits within

30 mi of the quadrangle that contain or are overlain by the

620 ka Lava Creek B volcanic ash.  The unit is tentatively

correlated with deposits in terraces T3 and T2 of Piety

(1981) along the Roaring Fork River between Glenwood Springs

and Carbondale and with deposits in terrace D of Bryant

(1979) farther upstream between Woody Creek and Aspen.

Exposed thickness commonly 6-30 ft and locally 70 ft along

the Colorado River, 30 ft along East Elk Creek, and 13 ft

along Garfield Creek; maximum thickness along the Colorado

possibly about 80 ft

Qfo   Older fan alluvium (late and middle Pleistocene)--

Mostly poorly sorted, clast- and matrix-supported, slightly

bouldery, pebble- and cobble-gravel in a silty sand matrix,

sandy pebble gravel, and pebbly sand.  Clasts are chiefly

angular to subrounded basalt and a minor amount of angular

to subrounded sandstone in deposits on the south side of the

Colorado River, and angular to subrounded sandstone in

deposits on the north side of the river.  Poorly bedded;

commonly contains discontinuous beds and lenses.  The unit

underlies slightly dissected surfaces that are mantled by

about 3-10 ft of loess (Qlo).  Deposited by Baldy Creek and

small intermittent streams that were graded to the tops of

terrace remnants composed of old terrace alluvium (Qto and

Qtt).  Locally includes valley-fill deposits of intermittent

streams, debris-flow deposits, sheetwash deposits (Qsw), and

colluvium (Qc).  Exposed thickness 6-35 ft; maximum

thickness possibly about 80 ft

Qp    Pediment deposits (middle Pleistocene)--Gravelly

alluvium and debris-flow deposits at four or more levels

that overlie gently sloping surfaces cut on Mancos Shale

(Km) and Wasatch Formation (Tw).  Locally as much as 20 ft

of relief on the pediment (bedrock surface) where it is

incised by stream channels.  Mostly poorly sorted, clast-

supported, bouldery, pebble- and cobble-gravel in a sandy

silt matrix and poorly sorted, cobbly, sandy pebble gravel

to pebbly silty sand.  Clasts are chiefly angular to

subrounded sandstone and locally minor amounts of siltstone,

limestone, and quartzite.  The quartzite clasts are

subrounded to rounded and were probably derived from older

fluvial surficial deposits.  The unit probably consists of

alluvium, debris-flow deposits, sheetwash deposits (Qsw),

and colluvium (Qc).  Nonsorted, bouldery, debris-flow(?)

deposits are common in the upper part of the unit.  Some of

the sandstone boulders in the unit near the Grand Hogback

are as long as 10 ft.  A stage III K soil horizon is locally

formed in the upper 20-30 in. of the unit and some of the

sandstone cobbles in the upper 6 ft are disintegrated and

weathered to sand-size particles.  The bedrock beneath the

unit is locally slightly oxidized to a depth of 10 ft or

more.  The unit is dissected and is mantled by about 6-13 ft

of loess (Qlo), which locally consists of two or more

sheets.  A reddish-yellow (5YR 6/6) argillic B horizon,

25 in. thick, is locally present in the top of the basal

loess sheet.  The lower limits of the pediment deposits are

about 130-330 ft above Elk and West Elk Creeks and about

130-620 ft above the Colorado River.  The three lower

pediment deposits along the Colorado River east of New

Castle appear to be graded to terrace remnants composed of

old terrace alluvium (Qto and Qtt) that are about 130-150,

220, and 250 ft above the river.  Some of the loess (Qlo) on

gently sloping surfaces along the Colorado River and on the

east side of Garfield Creek may overlie pediment deposits.

Exposed thickness commonly 6-15 ft; maximum observed

thickness 30 ft; maximum thickness possibly about 50 ft

Qbb   Basaltic boulder gravel (middle or early Pleistocene)-

Bouldery pediment(?) deposits consisting mainly of alluvium

and debris-flow deposits and possibly minor fan alluvium at

two levels that overlie gently sloping surfaces cut on

Wasatch Formation (Tw) in the southern part of the

quadrangle.  These surfaces are about 640-1,080 ft above

Divide Creek and are about 320-560 ft below the adjacent

geomorphic surface that is underlain by high-level basaltic

alluvium (QTba).  The unit is poorly exposed, but it appears

to consist of very poorly sorted, clast-supported, very

bouldery cobble gravel in a sand matrix.  Clasts are

subangular to subround basalt that are commonly greater than

3 ft in length and are as long as 6 ft.  The gravel is

locally mantled by about 3-5 ft of non-pebbly to pebbly,

slightly silty sand.  A stage III K soil horizon, 25 in.

thick, is formed in the top of the unit.  The K horizon is

mantled by loess (Qlo) that is at least 1.5 ft thick.  The

unit probably locally includes sheetwash deposits (Qsw).

Exposed thickness 13-20 ft; maximum thickness possibly about

60 ft

QTba  High-level basaltic alluvium (early Pleistocene or

late Pliocene)--Fan alluvium and valley-fill or pediment(?)

deposits that underlie gently sloping geomorphic surfaces in

the southern part of the quadrangle that are more than

900 ft above the Colorado River and about 960 ft above

Garfield Creek.  The northern part of the unit underlies a

gentle, west-sloping geomorphic surface.  It is a thick

(150 ft) sequence of alluvial-fan deposits that consist of

basaltic stream alluvium and debris-flow deposits.  These

deposits are well exposed in a steep landslide scarp

(NE1/4SE1/4 sec. 7, T. 6 S., R. 91 W.) where they consist of

alternating lenticular beds and lenses (about 0.3-6 ft

thick) of mostly poorly bedded, poorly sorted, clast-

support, very bouldery, cobble- and pebble-gravel and cobbly

pebble gravel in a matrix of slightly silty sand.  The

sequence also includes minor amounts of poorly sorted, sandy

pebble gravel, pebbly sand, and sand.  The clasts are mostly

subangular to rounded basalt and a minor amount of sandstone

and intrusive igneous rocks.  Basalt clasts are commonly

6 ft in length and are as long as 15 ft.  A soil K horizon,

48 in. thick, is formed in pebbly, sandy alluvium at the top

of the sequence.  The K horizon has stage IV carbonate

morphology in the upper 25 in. and stage III morphology in

the lower 23 in.  The K horizon is overlain by at least

18 ft of loess (Qlo), which consists of at least five

separate loess sheets, about 3-4 ft thick.  Where exposed,

the sequence of alluvial-fan deposits overlies Wasatch

Formation (Tw), but these deposits may locally overlie or be

interstratified with unexposed gravelly alluvium deposited

by the Colorado River.  The presence of underlying or

interstratified Colorado River alluvium is suggested by the

lithologic composition of the colluvial deposits derived

from the alluvial-fan deposits.  These colluvial deposits

locally contain a minor amount of rounded and well rounded

pebbles and cobbles that are identical to those in nearby,

but younger and topographically lower, terrace deposits that

were laid down by the Colorado River.  The southern part of

the unit underlies a gentle, north-sloping geomorphic

surface and is poorly exposed.  It appears to be either

valley-fill or pediment(?) deposits that consist of bouldery

alluvium and colluvium that accumulated on a former valley

floor or on a pediment(?) surface cut on the Wasatch

Formation.  The deposits are probably mostly poorly sorted,

clast-supported, bouldery, cobble- and pebble-gravel in a

sand matrix.  The clasts are mostly

subangular to rounded basalt that are as long as 6 ft.  The

gravelly deposits are overlain by one or more loess sheets

(Qlo).  The unit may have been deposited by ancestral

Garfield Creek and(or) Baldy Creek.  Exposed thickness of

fan alluvium in the northern part of the unit 150 ft;

thickness of valley-fill or pediment(?) deposits in the

southern part of the unit possibly as much as 80 ft

Alluvial and colluvial deposits-Sand and gravel in flood

plains and small alluvial fans along minor tributary

streams, and sheets of pebbly sand that locally overlie

alluvial deposits in valley bottoms and mantle the adjacent

valley sides and hill slopes

Qac   Undivided alluvium and colluvium (Holocene and late

Pleistocene)--Chiefly undifferentiated flood plain and

stream channel deposits (Qfp), young fan alluvium (Qfy), and

colluvial debris-flow (Qc) and sheetwash  (Qsw) deposits.

The alluvium typically consists of interbedded sand, pebbly

sand, and pebble gravel and ranges from thin-bedded

(0.2-6 in.) silty sand to thick-bedded (>3 ft), poorly

sorted, clast- and matrix-supported, slightly bouldery,

pebble- and cobble-gravel in a sand matrix.  The sheetwash

is typically pebbly silty sand.  Alluvial deposits form

flood plains, stream channels, and small alluvial fans along

Alkali Creek and the lower reaches of some of the larger

intermittent streams.  Sheetwash deposits locally mantle the

valley bottoms and the adjacent valley sides and hill

slopes.  Exposed thickness of the alluvium 3-26 ft; maximum

thickness probably about 40 ft.  Exposed thickness of the

colluvium 3-6 ft; maximum thickness probably about 15 ft

Colluvial deposits-Silt, sand, and gravel on valley sides

and hill slopes that were mobilized, transported, and

deposited by gravity and sheet erosion

Qc    Colluvium, undivided (Holocene and late Pleistocene)--

Mostly clast-supported, pebble, cobble, and boulder gravel

in a silty sand matrix, and gravelly, silty sand, sandy

silt, and clayey silt.  Deposits derived from the Mancos

Shale (Km) commonly contain more silt and clay than those

derived from the other bedrock units.  Deposits derived from

the Mancos Shale and shale in the Mesaverde Group (Kmv) may

contain expansive clays and have high shrink-swell

potential.  Deposits derived from the Eagle Valley Evaporite

(IPe) are prone to hydrocompaction and subsidence owing to

the dissolution of gypsum, anhydrite, and halite.  Typically

unsorted to poorly sorted and unstratified to poorly

stratified.  Clasts are typically angular to subrounded;

their lithologic composition reflects that of the bedrock

and(or) the surficial deposits from which the colluvium was

derived.  The unit locally includes sheetwash (Qsw) creep,

debris-flow, and landslide (Qls) deposits that are too small

to map separately or that lack distinctive surface

morphology and could not be distinguished in the field or on

aerial photographs.  The map unit also locally includes thin

loess (Qlo) mantles on older gently sloping colluvial

deposits, small deposits of alluvium and colluvium (Qac) in

and along minor drainageways, and probably small pediment

deposits (Qp) on the north side of the Grand Hogback.

Exposed thickness 6-13 ft; maximum thickness probably about

15 ft

Qsw   Sheetwash deposits (Holocene and late Pleistocene)--

Mostly pebbly, silty sand and sandy silt that are derived

chiefly from weathered bedrock and loess (Qlo) by sheet

erosion.  Common on gentle to moderate slopes in areas

underlain mostly by the Maroon Formation (IPm), Mancos Shale

(Km), and Wasatch Formation (Tw).  The unit locally includes

small deposits of loess (Qlo) and undivided alluvium and

colluvium (Qac) in and along minor drainageways; may locally

include creep (Colluvium, Qc) and alluvial-fan (Qfy)

deposits.  Exposed thickness 6-15 ft; maximum thickness

probably about 30 ft

Qls   Landslide deposits (Holocene and late Pleistocene)--

Chiefly unsorted and unstratified rock debris characterized

by hummocky topography.  Many of the landslide deposits are

complex (Varnes, 1978).  The younger deposits are commonly

bounded upslope by crescentic headwall scarps and downslope

by lobate toes.  The unit includes small debris-slide,

slump-earth-flow, earth-flow, and debris-flow deposits

(Varnes, 1978).  The sizes and lithologies of the clasts and

the grain-size distributions of the matrices of these

deposits reflect those of the bedrock units and surficial

deposits that were displaced by sliding.  Rock and evaporite

deposits of the Eagle Valley Evaporite (IPe) were mobilized

and incorporated in debris-slide deposits near the northeast

corner of the quadrangle.  Earth-slump and earth-flow

deposits are common near the southern quadrangle boundary

where they were derived from the Wasatch Formation (Tw) and

the overlying high-level basaltic alluvium (QTba).  The

deposits derived from the Eagle Valley Evaporite are prone

to hydrocompaction and subsidence owing to the dissolution

of gypsum, anhydrite, and halite.  The unit locally includes

sheetwash (Qsw), creep, and debris-flow (Colluvium, Qc)

deposits.  Exposed thickness 6-15 ft; maximum thickness

possibly 150 ft

Qta   Talus deposits (Holocene and late Pleistocene)--

Chiefly unsorted, crudely stratified, angular, cobbly and

bouldery rubble on steep slopes below outcrops of Dakota

Sandstone (Kd).  The matrix is mostly sand and silt; some of

it may be of eolian origin.  The upper part of the unit

locally lacks matrix.  The unit probably includes minor

deposits of colluvium (Qc).  Only one large deposit is

mapped in the northeast part of the quadrangle.  Small talus

deposits in the valleys of Elk Creek and Main Elk Creek were

not mapped.  Maximum thickness probably about 50 ft

Eolian deposits--Wind-deposited sand, silt, and clay that

mantles level to gently sloping surfaces

Qlo   Loess (late and middle? Pleistocene)--Wind-deposited,

nonstratified, friable, slightly plastic to plastic,

slightly clayey, sandy silt and silty sand.  The unit may

locally include minor deposits of clayey silt.  The grain-

size distribution of slightly weathered loess in and near

the quadrangle is about 23 percent sand, 55 percent silt,

and 17 percent clay (Harmon and Murray, 1985, tables 13 and

14).  Most of the sand-size particles are very fine and

fine.  The unit is prone to sheet erosion, gullying, and

compaction when wet.  Locally includes some loess-derived

sheetwash (Qsw) and creep (Colluvium, Qc) deposits.

Deposited during five or more episodes of eolian activity.

Deposition may have continued into Holocene time.  Probably

derived chiefly from flood plain sediments of the Colorado

River and its major tributaries and possibly in part from

(1) outcrops of Tertiary siltstone and mudstone in the

Piceance Basin west of the quadrangle (Tweto, 1979) and

(2) large areas of exposed sandstone in the Canyonlands

region in southeastern Utah (Whitney and Andrews, 1983).

The mapped distribution of loess is approximate, because it

lacks distinct topographic expression.  The unit commonly

mantles level to gently sloping surficial deposits that are

older than the younger fan alluvium (Qfy).  Younger terrace

alluvium (Qty) is mantled by one loess sheet, older terrace

alluvium (Qto) is locally mantled by two loess sheets,

pediment deposits (Qp) and the oldest terrace alluvium (Qtt)

are locally mantled by two or more loess sheets, and the

high-level basaltic alluvium (QTba) is mantled by five or

more loess sheets.  The soil that is formed in the upper

loess sheet on the older terrace alluvium commonly consists

of the following horizons:  an organic-enriched A horizon

about 12 in. thick; a cambic B horizon about 4 in. thick; a

weak to moderate prismatic, argillic B horizon about 16 in.

thick; and a stage I Bk horizon greater than 30 in. thick.

The soil that is formed in the lower loess sheet on the

older terrace alluvium contains more clay and calcium

carbonate than the soil in the upper loess sheet and

commonly consists of the following horizons:  a cambic B

horizon about 8 in. thick; a strong prismatic, argillic B

horizon about 16 in. thick that contains weak stage II

calcium carbonate; a weak stage III K horizon about 20 in.

thick; and a stage I-II Bk horizon about 12 in. thick.

Where the upper loess sheet is composed of

silty sand, the soils have weakly developed, non-prismatic

argillic B horizons that are about 14 in. thick.  The upper

two loess sheets on the high-level basaltic alluvium have

10YR-hue colors and cambic and(or) incipient argillic B

horizons, whereas the lower three loess sheets have 7.5YR-

hue colors and the lowest two loess sheets have strong

prismatic argillic B horizons, 20-35 in. thick, that have

stage I-II calcium carbonate morphology.  The degree of soil

development in these loess sheets suggests that the upper

two sheets are probably late Pleistocene and one or more of

the lower three sheets may be middle Pleistocene.  Exposed

thickness 5-18 ft; commonly 6-13 ft thick

Tw    Wasatch Formation (Eocene to Paleocene)--Interbedded,

variegated reddish-brown, reddish-purple, yellowish-brown,

tan, and white conglomerate, conglomeratic sandstone,

sandstone, siltstone, mudstone, and claystone unconformably

overlying Mesaverde Group rocks. Sediment is first cycle,

poorly sorted, and contains an abundance of metamorphic and

granitic rock fragments; locally includes a basal cobble

conglomerate as much as about 50 ft thick, designated the

Ohio Creek Conglomerate by Bass and Northrop (1963, p.J58).

Coarse clastic beds are trough crossbedded and vary in

thickness, whereas fine-grained clastic beds are commonly

thin.  All beds are laterally discontinuous and, in general,

are lenticular; overall, the sequence fines upward.  About

5000 ft thick

Deposition of sediments occurred in a high-

energy stream-dominated fluvio-lacustrine depositional

setting during the initial phase of sedimentary infill of

the Piceance basin as it formed during the Laramide orogeny.

Sediments were derived form multiple source areas within the

present Rocky Mountains as they were uplifted during the

orogeny.  Braided stream and flood plain deposition

dominated as the primary means of sediment transport and

deposition.  Differentiation of sediment into more discrete

lithofacies within alluvial complexes was hindered by the

relatively short distances from sediment source to

depocenters in the basin and the apparent large volume of

sediment.

Kmv   Mesaverde Group rocks undivided (Upper Cretaceous)--

Thin and thick beds of yellowish-brown and olive-gray

carbonaceous mudstone interbedded with fine- to medium-

grained silty sandstone, siltstone, and claystone; contains

thin beds and pockets of silty and sandy pebble- and cobble-

conglomerate; beds are generally massive.  Locally

interbedded with thin beds (seams) and laminations of coal;

clinker is common.  Contains several beds of white, well-

sorted, short forset-crossbedded, quartzose sandstone in the

lower part which contrasts strongly in appearance and

physical character with surrounding silty carbonaceous beds.

Except for white sandstone beds, in clastic sediments are

generally poorly sorted, subangular, quartzose, and occur

with an appreciable quantity of particulated coaly

carbonaceous material; some beds are silica cemented; beds

form prominent ridges and cliffs.  About 5000 ft thick

Deposition occurred during the first major

Cretaceous regressive marine cycle in the Rocky Mountain

seaway following the Dakota transgressive cycle.  Mesaverde

deposition occurred dominantly in backshore areas and on

deltas of the lower coastal plain where sediment-laden

streams meandered between coal swamps, marshes, and mudflats

behind a seaward-migrating shoreline.  White sandstone beds

preserved in the lower part of the sequence are the product

of shoreline processes at the beach where surf- and wave-

zone fluvial activity winnowed, sorted, and otherwise

concentrated sand-sized particles.

Mineable coal beds (not mapped) in the

Mesaverde occur in three zones in the map area; the lower

zone consists of the Wheeler, D, and Allen coal beds located

approximately at 900, 985, and 1500 ft, respectively, above

the base of the unit.  The middle zone consists of the C, B,

and A coal beds located approximately at 2200, 2245, and

2330 ft, respectively, above the base of the unit.  The

upper zone consists of a single bed, the Keystone coal bed,

located at about 3900 ft above the base of the unit (Gale,

1910; Bass and Northrop, 1963, p.J57).

Km    Mancos Shale (Upper Cretaceous)--Dominantly light- to

dark-gray carbonaceous shale locally containing thin

lenticular beds of dark-gray and black fossiliferous (mainly

fragmented) limestone and thin-bedded, very fine grained

silicious silty sandstone; sediments of the Mancos Shale are

generally limy.  The formation contains white to yellowish-

brown bentonite horizons (altered volcanic ash) a few inches

thick.  The upper and lower formation contacts of the Mancos

are conformable. Unit about 5,500 ft thick.  Generally

underlies floors of valleys where it commonly is poorly

exposed beneath Quaternary surficial deposits.

Deposition occurred primarily on the

continental slope in transgressive (lower part of the

sequence) and regressive (upper part of the sequence)

submarine environments. Clastic deposition occurred by

sediment settling and turbidity flow, whereas limestone

formed by chemical precipitation.  The dark-gray and black

color of the rocks is attributed largely to the content of

black coaly detrital organic matter apparently derived from

the destruction of pre-Dakota coaly carbonaceous swamp

deposits on the lower coastal plain by high-energy

transgressive shoreline processes.

Kd    Dakota Sandstone (Lower Cretaceous)--Yellowish-brown,

medium- to coarse-grained, massive to crossbedded, quartzose

sandstone containing pockets and lenses of gray chert-pebble

and chert-cobble  conglomerate; interbedded with dark-gray

to black carbonaceous sandy siltstone and mudstone.

Sandstone is commonly well sorted, angular, and well

cemented with silica; forms prominent cliffs.  Contact with

underlying Morrison Formation is unconformable; contact with

overlying Mancos Formation is conformable and locally

intertongueing.  About 175 ft thick

Deposition occurred on a lower coastal

plain, at or near the shoreline, and in shallow marine

embayments in a transgressive coastal setting.  Sandstone

and conglomerate were deposited in broad, distributary

fluvial channels in which fluvial currents and, locally,

offshore currents, influenced the structure and distribution

of sand bodies.  Intervening carbonaceous siltstone was

deposited in mudflats, bays, and estuaries adjacent to the

shoreline and in interfluvial areas adjacent to distributary

channels in backshore areas.  Much silt and carbonaceous

material deposited in shallow water embayments and estuary

environments as well as farther seaward, may have been

derived from the destruction of backshore swamp and marsh

deposits as the high-energy shoreline environment migrated

landward during the Dakota transgressive cycle.

Jm    Morrison Formation (Upper Jurassic)--Medium- to light-

green and maroon shale and mudstone and thin beds of silty

sandstone (mainly in lower part) and dark-gray limestone.

Sand fraction is mainly clear, gray, and white quartz

grains; but, green, gray, and brown chert grains are common.

Beds are thin and lenticular.  About 500 ft thick

Deposition occurred in a lacustrine-

dominated fluvio-lacustrine depositional setting.  In the

map area, the formation is a single lithofacies of a large

fluvio-lacustrine system present throughout much of the

Colorado Plateau and western interior of the United States.

Je    Entrada Sandstone (Upper Jurassic)--Light-orange,

medium-to very fine grained, well-sorted, crossbedded

sandstone; sand grains are subrounded to well rounded and

consist mainly of quartz.  Contact with overlying Morrison

Formation is sharp and conformable; contact with underlying

Chinle Formation is unconformable.  About 90 ft thick

Crossbed sets are large scale and apparently

were formed by eolian activity in large, laterally

extensive, dune fields.  The basal few inches of the

formation commonly consists of a layer of coarse-grained

sand- and pebble-sized clasts of variegated chert and

quartz.  This layer apparently formed as a lag concentrate

by wind deflation on the erosion surface developed on top of

the underlying Chinle Formation.  The formation is present

throughout much of the Colorado Plateau and western interior

of the United States.

TRc   Chinle Formation (Upper Triassic)--Thinly and even-

bedded red beds composed of shale and siltstone and thin

beds of limestone and limestone-pebble conglomerate.  Shale

and siltstone are dark reddish brown to reddish orange;

limestone and limestone-pebble conglomerate are light

purplish red and gray.  Shale and siltstone locally exhibit

ripple marks and mudcracks.  Contact with the State Bridge

Formation below is gradational; contact with the Entrada

Sandstone above is unconformable.  About 300 ft thick

Grain size of clastic units, primary

sedimentary structures, and the presence of limestone-pebble

conglomerate, suggest that deposition occurred in relatively

shallow, seasonally dry, lacustrine depositional settings

within large interfluvial areas (flood plains) or in broad

shallow lakes which occupied the central part of the

sediment-filled Eagle basin.  The lack of coarse-grained,

bed-load clastic sediment in association with the formation

is characteristic.  The formation is present throughout the

southwestern United States and is represented in the map

area by only a part of the distal, fine-grained lithofacies

of the formation.

TRPsb  State Bridge Formation--(Lower Triassic? and Permian)

Thin uniform beds of brick-red, reddish-brown, light-gray,

and green and greenish-gray micaceous siltstone and shale

interbedded locally with medium- to fine-grained crossbedded

sandstone and thin beds of gypsum/anhydrite. Unit grades

vertically into underlying and overlying formations.  About

150-200 ft thick

The unit represents a seasonally active

lacustrine-dominated fluvio-lacustrine sequence deposited in

the latter stage of Eagle basin sedimentary infill at a time

when lakes and ponds were numerous and streams feeding them

were shallow, sluggish, and near base level within the

basin.

PIPm  Maroon Formation (Permian and Pennsylvanian)--

Principally red beds of conglomerate, conglomeratic

sandstone, arkosic sandstone, siltstone, mudstone,

claystone, and shale and minor thin beds of limestone.

Conglomerate contains pebble- and cobble-sized material in a

matrix of poorly sorted, fine- to medium- and very coarse

grained angular sand.  Sediments are first cycle; they

contain appreciable mica; colors are dominantly bright

reddish orange and reddish brown; limestone beds are dark

gray.  Beds are generally trough crossbedded and uniform in

thickness; they range from a few inches to several tens of

feet in thickness and rarely exhibit scoured bases.  Fine-

grained beds locally exhibit current and oscillation ripple

marks and mudcracks.  About 3,000 ft thick

Deposition occurred dominantly in braided

streams and adjacent flood plains in the mid-fan area of a

large coalescing, arid to semi-arid, alluvial-fan complex

present in marginal areas of the Paleozoic Eagle basin.

The upper part of the formation in the map

area includes possible stratigraphic equivalents of the

Weber Sandstone of northeast Utah and northwest Colorado

(Bass and Northrop, 1963, p.J47).  Overlying the Weber,

ranging from about 50 to 100 ft below the top of the

formation is a 6 foot interval of fossiliferous dolomite and

dolomitic limestone differentiated by past workers and named

the South Canyon Creek Member of the Maroon Formation (Bass

and Northrop, 1963, p J48).

IPe   Eagle Valley Evaporite (Middle Pennsylvanian)--

Principally gypsum and anhydrite and lesser amounts of

halite; contains traces of potash salts.  Evaporites contain

interbeds of conglomerate, sandstone, siltstone, shale, and

limestone.  Beds range from a few feet to about 150 ft in

thickness.  Colors are grayish white, yellowish gray, black,

dark gray; beds are intensely folded, faulted, and ductilely

deformed by load metamorphism, diapiric upwelling, and

flowage and hydration of anhydrite (Mallory, 1971).

Intertongues with Belden Formation below and the Maroon

Formation above; intertongues laterally with the Minturn

Formation east of the map area.  Formation incompletely

exposed in the map area.  Estimated total thickness 1000-

1500 ft

Evaporites are the products of sea water

evaporation in a restricted seaway that was present in the

central part of the landlocked Paleozoic Eagle basin which

formed between the Uncompaghre and Front Range uplifts

(Mallory, 1971).  The formation is a distal lithofacies of a

large fluvio-lacustrine and marine depositional system

present in the basin from early Pennsylvanian through early

Triassic time.  Associated fluvial and lacustrine clastic

sediments in the Eagle Valley Evaporite sequence are

products of transgressive and regressive fluvial and

lacustrine deposition under arid to semi-arid climatic

conditions adjacent to the seaway.

REFERENCES CITED

American Geological Institute, 1982, Grain-size scales used

by American geologists, modified Wentworth scale, in Data

sheets (2nd ed.): Falls Church, Va., American Geological

Institute, sheet 17.1.

Bass, N.W., and Northrop, S.A., 1963, Geology of Glenwood

Springs quadrangle and vicinity, northwestern Colorado:

U.S. Geological Survey Bulletin 1142-J, 74 p.

Bryant, Bruce, 1979, Geology of the Aspen 15-minute

quadrangle, Pitkin and Gunnison Counties, Colorado:  U.S.

Geological Survey Professional Paper 1073, 146 p.

Colman, S.M., 1985, Map showing tectonic features of late

Cenozoic origin in Colorado:  U.S. Geological Survey

Miscellaneous Geologic Investigations Series Map I-1556,

scale 1:1,000,000.

Gale, H.S., 1910, Coal fields of northwestern Colorado and

northeastern Utah: U.S. Geological Survey Bulletin 415, pl.

10, 265 p.

Gile, L.H., Peterson, F.F., and Grossman, R.B., 1966,

Morphological and genetic sequences of carbonate

accumulation in desert soils:  Soil Science, v. 101,

p. 347-360.

Green, Morris W., Fairer, George M., and Shroba, Ralph R.,

1993, Preliminary geologic map of the New Castle quadrangle,

Garfield County, Colorado: U.S. Geological Survey Open-File

Report 93-310, 33 p., 1 pl., scale 1:24,000.

Guthrie, R.L., and Witty, J.E., 1982, New designations for

soil horizons and layers and the new Soil Survey Manual:

Soil Science Society of America Journal, v. 46, p. 443-444.

Harmon, J.B., and Murray, D.J., 1985, Soil survey of Rifle

area Colorado -- parts of Garfield and Mesa Counties:  U.S.

Department of Agriculture, Soil Conservation Service, 149 p.

Izett, G.A., and Wilcox, R.E., 1982, Map showing localities

and inferred distributions of the Huckleberry Ridge, Mesa

Falls, and Lava Creek ash beds (Pearlette family ash beds)

of Pliocene and Pleistocene age in the western United States

and southern Canada:  U.S. Geological Survey Miscellaneous

Investigations Series Map I-1325, scale 1:4,000,000.

Larson, E.E., Ozima, Minoru, and Bradley, W.C., 1975, Late

Cenozoic basic volcanism in northwestern Colorado and its

implications concerning tectonism and the origin of the

Colorado River system, in Curtis, B.F., ed., Cenozoic

history of the southern Rocky Mountains:  Geological Society

of America Memoir 144, p. 155-178.

Mallory, W.M., 1971, The Eagle Valley Evaporite, northwest

Colorado -- a regional synthesis: U.S. Geological Survey

Bulletin 1311-E, 37 p.

Munsell Color, 1973, Munsell soil color charts:  Baltimore,

Md., Kollmorgen Corp., Macbeth Division.

Pierce, K.L., 1979, History and dynamics of glaciation in

the northern Yellowstone National Park area:  U.S.

Geological Survey Professional Paper 729-F, 90 p.

Pierce, K.L., Obradovich, J.D., and Friedman, Irving, 1976,

Obsidian hydration dating and correlation of Bull Lake and

Pinedale glaciations near West Yellowstone, Montana:

Geological Society of America Bulletin, v. 87, n. 5,

p. 703-710.

Piety, L.A., 1981, Relative dating of terrace deposits and

tills in the Roaring Fork Valley, Colorado:  Boulder,

University of Colorado, M.S. thesis, 209 p.

Richmond, G.M., 1986, Stratigraphy and correlation of

glacial deposits of the Rocky Mountains, the Colorado

Plateau, and the ranges of the Great Basin, in Sibrava, V.,

Bowen, D.Q., and Richmond, G.M., eds., Quaternary

glaciations in the northern hemisphere:  Quaternary Science

Reviews, v. 5, p. 99-127.

Richmond, G.M., and Fullerton, D.S., 1986, Introduction to

Quaternary glaciations in the United States of America, in

Sibrava, V., Bowen, D.Q., and Richmond, G.M., eds.,

Quaternary glaciations in the northern hemisphere:

Quaternary Science Reviews, v. 5, p. 3-10.

Shroba, R.R., 1989, Physical properties and laboratory data

for soils formed in Pleistocene tills at Bull Lake, Dinwoody

Lakes, and Fremont Lake, Fremont and Sublette Counties,

Wyoming:  U.S. Geological Survey Open-File Report 89-370,

14 p.

Soil Survey Staff, 1975, Soil taxonomy:  U.S. Department of

Agriculture Handbook 436, 754 p.

Stover, B.K., and Soule, J.M., 1985, Surficial geologic map,

in Surficial geology, geomorphology, and general engineering

geology of parts of the Colorado River valley, Roaring Fork

Valley, and adjacent areas, Garfield County, Colorado:

Colorado Geological Survey Open File Report 85 1, scale

1:50,000.

Tweto, Ogden, 1979, Geologic map of Colorado:  U.S.

Geological Survey, scale 1:500,000.

Varnes, D.J., 1978, Slope movement types and process, in

Schuster, R.L., and Krizek, R.J., eds., Landslides,

analysis, and control:  National Academy of Sciences,

Transportation Research Board Special Report 176, p. 11 33.

Wentworth, C.M. and Fitzgibbon, T.T., 1991, ALACARTE users

manual: U.S. Geological Survey Open-File Report 91-587C, 268

p.

Whitney, J.W., and Andrews, E.D., 1983, Past and present

geomorphic activity in the Piceance Creek drainage basin,

northwestern Colorado, in Gary, J.H., ed., Sixteenth Oil

Shale Symposium Proceedings:  Golden, Colorado School of

Mines Press, p. 566 577.

Whitney, J.W., Piety, L.A., and Cressman, S.L., 1983,

Alluvial history in the White River basin, northwest

Colorado [abs]:  Geological Society of America Abstracts

with Programs, v. 15, n. 5, p. 328.

CONVERSION FACTORS

------------------------------------------------

Multiply             By            To obtain

-------------------------------------------------

inches (in.)         2.540         centimeters(cm)

feet (ft)            0.3048        meters (m)

miles (mi)           1.609         kilometers (km)

-------------------------------------------------

Refer to non-digitized version of this open-file report for

a description of map symbols.